ECE 5340/6340 APPLICATION OF FDFD TO TRANSMISSION LINE IMPEDANCE

Transmission line theory:

- Can’t use Lumped element approach because wavelength is approx. size of the FDFD element.

- Use incremental approach instead

Zo=LC=LCoCCo=1VpoCCoVp=1LC=VpoCoCVpo=1LoCo=2.996e8ms

For TL in free space Vp=Vpo.

Co = Capacitance/unit length of TL without dielectric

C  = Capacitance/unit length of TL with dielectric

TO CALCULTE CAPACITANCE from numerical simulation:

C=qVq=e.E·dS=e(-Ñf)·dS=e−dfdndS

Use Numerical differentiation and integration:

dV/dn = (Vn+1 + Vn ) / dn

trapezoidal integration = h [ f(a)/2 + f(b)/2 + sum(f)]

dV/dn = (Vj,2 - Vj,1)/hx

e¶f¶n·dl1=hye1f21−f11hx+f22−f12hx+f23−f13hx

METHOD TO FIND characteristic impedance Zo:

- Simulate Microstripline (complete, with two dielectrics)

a ) Find potential (phi) distribution

- Integrate 4 line integrals around the top conductor

to find charge on top conductor.  (May use symmetry.)

- Find Q1 and C1

- Simulation Microstripline (air ONLY, no dielectrics)

- Find potential (phi distribution)

- Integrate 4 line integrals around the top conductor

to find charge on top conductor.  (May use symmetry.)

- Find Qo and Co

- Calculate Vp and Zo:

Vp = Vo  (Co/C)

Zo = 1/ (Vpo (C Co))