ECE 5340/6340

FINITE-DIFFERENCE FREQUENCY-DOMAIN METHOD

- (FD-FD or FD method)

- Used to solve ordinary partial differential equations (ODEs)

Laplace'sEquation:Ñ2F=0Poisson'sEquation:Ñ2F=-qveHelmholtzEquation:(Ñ2+ke2)F=0

Laplace’s Equation used for electrostatics, TM lines

Poisson’s Equation used for semiconductors

Helmholtz Equation used for waveguides

FDTD METHOD:

Differential  Difference System Solve Values at

Equation          Equation       of linear                 discrete points

equations

NUMERICAL DIFFERENTIATION

¶f¶x=f'=slopeofcurvefi'@fi+1−fihForwardDifferencefi'@fi−fi−1hBackwardDifferencefi'@fi+1−fi−12hCentralDifference

TYPES OF ERROR:

- Discretization Error:  h is too large to represent derivative

- Round-off Error: h is too small.  Difference (a-b) is so small that computer has roundoff error.

Optimal value of h is approx. wavelength/10 to wavelength/50

For most simulations.

ERROR ANALYSIS of NUMERICAL DIFFERENTIATION (Digitization error):

Taylor expansion:

- fi+1 = fi  + h fi’ + (h2/2) fi’’+ … Forward Taylor Series

fi’= fi+1 - fi  - (h/2) fi’’+ …    Forward Difference Equation

h        ^^Error on the order of: (h/2) fi’’ (overestimate)

- fi-1 = fi  - h fi’ + (h2/2) fi’’- (h3/6) fi’’’ +  … Backward Taylor Series

fi’= fi - fi-1  + (h/2) fi’’+ …    Backward Difference Equation

h        ^^Error on the order of: (h/2) fi’’ (underestimate)

Take (1) - (2):

fi+1 - fi-1 = 2h fi’ + (h3/3) fi’’ + …

fi’= fi+1 - fi-1  - (h2/6) fi’’’   Central Difference Equation

2h        ^^Error on the order of: (h2/6) fi’’’ (underestimate)

EXAMPLE:

f(x) = x2   f’(x) = 2x    f’’(x) = 2

f(x) = [4  9 16]

x    = [2  3  4]

h = 0.5  because fi+1 = 3 and fi-1 = 2  (2h = 3-2)

f’(x=2.5) = (9-4)/1 = 5

f’(x=3.5) = (16-9)/1 = 7

Error = h2 f’’’ / 6 = 0  (predicted)

= 0 (observed)

2nd DERIVATIVE (which is required by 2)

f’(xi + h/2) = (fi+1 - fi) / h  + error ((h2/ 6)  f’’’)

f’(xi - h/2) = (fi - fi-1) / h  + error ( (h2/ 6 ) f’’’)

f’’(xi) = (f’(xi + h/2) - f’(xi - h/2)) / h

= (fi+1 - 2fi + fi-1 ) / h2 + error ((h2/ 3 ) f’’’)

Note:  These error terms are ORDER OF THE ERROR, because all higher-order (higher orders of h) error terms were neglected.

Example (continued)

h = 1 in this case, because fi+1 = 4 and f i-1 = 2 (2h = 4-2)

f’’(x=3) = (f’(x=3.5) - f’(x=2.5))/ (2*1) = (7 - 5) / 1 =2

OR:  = (16 - 2(9) + 4) / (12) = 2

Error = 0 because f’’’ = 0

FDFD METHOD:

Application of numerical differentiation to 2D Laplace Equation

¶2f¶x2+¶2f¶y2=0

5-point stencil commonly used for FDFD:

Ñ2f=¶2f¶x2+¶2f¶y2=0¶2f¶x2=fi+1,j−2fi,j+fi−1,jh2¶2f¶y2=fi,j+1−2fi,j+fi,j−1h2Ñ2f=fi+1,j+fi−1,j+fi,j+1+fi,j−1−4fi,jh2

This is the 5-point Difference Operator for Equi-spaced Data (hx=hy=h) … Equal-Arm Star, or Uniform 5-point stencil

Example 1: Rectangular Duct

Find the potential distribution.

Solution should satisfy Laplace’s Equation:

¶2f¶x2+¶2f¶y2=0

Subject to the BOUNDARY CONDITIONS:

f(x,0)=f(x,10)=f(0,y)=0f(20,y)=100volts

- Replace differential equation by difference operator

- where ij are discrete values of potential at points (nodes) within domain of interest

- Lay out coarse mesh

- h=20 cm / 4 divisions = 5 cm = 0.05 meters

- Apply Difference Equation at Each Node (1,2,3)

- Node 1: [0+0+0+P2 - 4*P1] / 0.05 2 = 0

- Node 2: [ P1+0+0+P3-4*P2]/ 0.05 2 = 0

- Node 3: [P2+100+0+0-4*P3]/ 0.05 2 = 0

- (Multiply through by 0.052)

- Solve:

−4101−4101−4P1P2P3=00−100 Gives:

- P1 = 1.786, P2 = 7.143, P3 = 26.786

- Repeat for Refined Mesh:  h=2.5 cm

- Again write 21 equations in 21 unknowns

Standard FDFD matrix for equal-arm star and Laplace’s equation: a -4 appears on diagonal, and a “1” appears for each neighboring element.  This is a banded matrix, 21x21.

Array of unknowns, 21x1

b vector: represents boundary conditions … -100 touches P7, P14, P21.  All others are 0.

- Solve:

h=5cm		h=2.5cm		Analyt.

Value    Err		Value  Err	Value

P1	1.786	-.692	1.289   -.195  1.0943

P2   7.143     -1.655    6.019   -.531   5.4885

P3   26.786   -.692      26.289  -.195   26.0944

Conclusion:

FDFD operator can be used to solve Laplace, Poisson, Helmholtz equations.

Error is caused by linear approximation to derivative and is on the order of h2 f''' for equal-spaced points.  Unequal-spaced points can be used, but the error will then be on the order of hf''.

FDFD results in a banded matrix with "4"s on the diagonal, "1" on neighboring elements.  The "b" matrix has zeros except where non-zero boundary conditions exist.