Time Domain Forms:

- Continuous Wave (CW)

Einc(t) = sin(t)

b) Ramped CW

Einc(t) = ramp(t) * sin(t)

- Pulsed

- Raised Cosine

Einc(t) = 1-cos(t)   0  t  1/Fmax

0	else

Fmax = ½ magnitude point (not ½ power)

- Gaussian

Einc(t) = exp( (to-t) / 2)

- Rectangular

- Other

Issues:

- CW is single frequency

- multiple frequency runs require additional FDTD simulation for each freq. Of interest

- this may or may not be the most efficient method, depending on the signal processing you use

- Pulsed gives multiple frequency info. With single run

- However… it requires DFT/FFT, which is not the most efficient method

- requires model with same dx as highest frequency CW run.

- Frequency dependent media

- Aliasing?  No.  Numerical dispersion eliminates too-high freqs.

- Signal processing

CONVERGENCE  (when to stop)

a ) CW:  When steady-state is reached.  Depends on # of internal oscillations (4-5 cycles is common)

- Pulsed:  When all fields have attenuated to zero.  Requires same # of cycles as CW with lowest freq. Of interest.

TYPES OF SOURCES:

- Forced – Localized

- Ez(I,j,k) = sin(t)

- Examples:  feedpoint of antenna, feed under microstripline, waveguide feed

- Can force E,H,I  = Hdl  H

- Beware:  Acts like a reflector

- Added Source (equivalent current source)

Ms=- {nxEiJs=nxHiÑxE=-m¶H¶t−MÑxH=e¶E¶t+J

Plane wave source done this way

Describe total and scattered field regions.